# Topic

## Engineering Department Office

#### Computer Networks

Academic Year: 2026-2027

Kavish Batavia (34) - 125BTCM2010

TYCM-3

Computer Engineering

## Shah and Anchor Kutchhi Engineering College

## 1. Objective

The objective of this project is to study the networking requirements of an organization with 15 users and develop a conceptual Local Area Network (LAN) design that provides reliable communication, resource sharing, centralized access to services, and secure connectivity to external networks.

The proposed network will use a single-domain environment with structured IP addressing and a Star topology. The design will include appropriate networking devices such as a router, switch, firewall, patch panel, and network rack.

## 2. Requirement Analysis

### 2.1 Organizational Requirements

The organization requires a reliable and scalable network to connect 15 users. The network should allow users to communicate with each other, share files and resources, access network services, and connect securely to external networks such as the Internet.

The main requirements are:

| Requirement | Description |
| --- | --- |
| Number of users | 15 users |
| Network type | Local Area Network (LAN) |
| Domain | Single-domain environment |
| IP addressing | IPv4 private addressing |
| Topology | Star topology |
| Central device | Managed Ethernet switch |
| External connectivity | Router connected to WAN/Internet |
| Security | Firewall for traffic filtering and protection |
| Physical organization | Patch panel and network rack |
| Scalability | Ability to add additional users/devices |

The network should also provide good performance, easy troubleshooting, centralized connectivity, and room for future expansion.

### 2.2 Number of Users

The organization has 15 users, including employees who require access to shared network resources and external services.

Each user will normally have a computer connected to the LAN through an Ethernet connection. A 24-port switch can be selected even though only 15 ports are initially required. This provides additional ports for future users, printers, access points, servers, or other network devices.

### 2.3 Single Domain Environment

The proposed network will operate in a single-domain environment. A single domain provides centralized management of user accounts, computers, authentication, and network resources.

For example, an organization could use a domain such as:

engineering.local

A domain-based environment allows administrators to manage users and computers from a central location. It also simplifies access control and application of organizational policies.

A dedicated domain controller/server may be added to the network if Active Directory or another directory service is required.

## 3. IP Addressing Plan

The organization will use private IPv4 addressing for internal communication.

A suitable network is:

Network: 192.168.10.0/27
Subnet Mask: 255.255.255.224
Usable Address Range: 192.168.10.1 – 192.168.10.30
Broadcast Address: 192.168.10.31

A /27 network provides 30 usable host addresses, which is sufficient for 15 users and network infrastructure while leaving room for future expansion.

### Proposed IP Address Allocation

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| Router LAN Interface | 192.168.10.1 | 255.255.255.224 | — |
| PC-01 | 192.168.10.2 | 255.255.255.224 | 192.168.10.1 |
| PC-02 | 192.168.10.3 | 255.255.255.224 | 192.168.10.1 |
| PC-03 | 192.168.10.4 | 255.255.255.224 | 192.168.10.1 |
| PC-04 | 192.168.10.5 | 255.255.255.224 | 192.168.10.1 |
| PC-05 | 192.168.10.6 | 255.255.255.224 | 192.168.10.1 |
| PC-06 | 192.168.10.7 | 255.255.255.224 | 192.168.10.1 |
| PC-07 | 192.168.10.8 | 255.255.255.224 | 192.168.10.1 |
| PC-08 | 192.168.10.9 | 255.255.255.224 | 192.168.10.1 |
| PC-09 | 192.168.10.10 | 255.255.255.224 | 192.168.10.1 |
| PC-10 | 192.168.10.11 | 255.255.255.224 | 192.168.10.1 |
| PC-11 | 192.168.10.12 | 255.255.255.224 | 192.168.10.1 |
| PC-12 | 192.168.10.13 | 255.255.255.224 | 192.168.10.1 |
| PC-13 | 192.168.10.14 | 255.255.255.224 | 192.168.10.1 |
| PC-14 | 192.168.10.15 | 255.255.255.224 | 192.168.10.1 |
| PC-15 | 192.168.10.16 | 255.255.255.224 | 192.168.10.1 |

Addresses from 192.168.10.17 to 192.168.10.30 remain available for future devices such as printers, servers, wireless access points, or additional computers.

## 4. Topology Selection

### 4.1 Selected Topology: Star Topology

A Star topology has been selected for the proposed network. In this arrangement, all computers and network devices are connected to a central switch.

### Basic Structure

Internet / WAN
↓
Firewall
↓
Router
↓
Switch
↓
PC-01, PC-02, PC-03, ... PC-15

The switch acts as the central connection point for the users.

### 4.2 Justification for Star Topology

Star topology is suitable for this organization for several reasons:

### Easy Management

All devices connect to a central switch, making the network easier for an administrator to monitor and manage.

### Easy Troubleshooting

If one computer or cable fails, the other computers can continue operating. The failed connection can be identified and repaired without affecting the entire LAN.

### Good Performance

Each computer has a dedicated connection to the switch. This provides better network performance compared with older shared-media topologies.

### Scalability

Additional computers can easily be connected to unused switch ports. Using a 24-port switch gives the organization room for future expansion.

### Reliability

A failure of one workstation cable normally affects only that workstation. It does not bring down the entire network.

### Suitable for Structured Cabling

Star topology works well with Ethernet cabling, patch panels, and network racks, making it appropriate for an organized office environment.

## 5. Key Networking Concepts

### 5.1 LAN

A Local Area Network (LAN) connects computers and network devices within a limited geographical area such as an office, department, school, or building.

In this project, the 15 user computers form the organization's LAN. The LAN allows users to communicate and share resources such as files, printers, and network services.

### 5.2 WAN

A Wide Area Network (WAN) connects networks across larger geographical areas. The Internet is an example of a WAN.

In the proposed design, the router provides connectivity between the organization's internal LAN and an external WAN/Internet connection.

LAN → Router → WAN/Internet

The LAN is used for internal communication, while the WAN provides access to external networks and services.

### 5.3 Physical Topology

Physical topology describes the actual physical arrangement of network devices and cables.

For this project, the physical topology is a Star topology. The computers are physically connected to a central switch using Ethernet cables.

A patch panel and rack can be used to organize the physical connections.

### 5.4 Logical Topology

Logical topology describes how data moves between devices over the network, regardless of their physical arrangement.

In the proposed network, Ethernet frames from one computer are forwarded through the switch to the appropriate destination. When communication is required with an external network, packets are forwarded to the router through the default gateway.

Example:

PC-01 → Switch → PC-02

For external communication:

PC-01 → Switch → Router → Firewall/WAN → Internet

## 6. Network Devices and Their Roles

### 6.1 Router

The router connects the organization's LAN to external networks.

Main functions include:

- Connecting LAN and WAN networks

- Routing IP packets

- Acting as the default gateway

- Providing communication between different networks

- Supporting future WAN/Internet connectivity

The proposed LAN interface can use:

192.168.10.1/27

### 6.2 Switch

The switch is the central device of the LAN.

Its functions include:

- Connecting the 15 user computers

- Forwarding Ethernet frames

- Providing dedicated switch ports to users

- Supporting efficient communication within the LAN

- Providing additional ports for future expansion

A 24-port switch is recommended because it provides more ports than the current requirement of 15 users.

### 6.3 Firewall

The firewall provides network security between the organization's internal network and external networks.

Its functions include:

- Filtering network traffic

- Blocking unauthorized connections

- Controlling access between trusted and untrusted networks

- Helping protect internal systems from external threats

- Supporting security policies

The firewall can be positioned between the internal network/router and the external network depending on the final network architecture.

### 6.4 Patch Panel

A patch panel provides an organized termination point for Ethernet cables.

Its advantages include:

- Organized cable management

- Easier identification of connections

- Simplified maintenance

- Reduced cable clutter

- Easier future expansion

The permanent cables from office network outlets can terminate at the patch panel, with short patch cables connecting the patch panel to switch ports.

### 6.5 Network Rack

The network rack provides a secure and organized physical location for networking equipment.

The rack can contain:

- Switch

- Patch panel

- Router

- Firewall

- Cable management equipment

- Power equipment where required

Using a rack improves organization, maintenance, physical security, and cable management.

## 7. Proposed Network Design Approach

The proposed network will use a hierarchical and structured design.

### Step 1: User Connectivity

Each of the 15 computers will be connected using Ethernet cables.

### Step 2: Patch Panel

Network cables from user locations will terminate at a patch panel in the network rack.

### Step 3: Switch Connectivity

Patch cables will connect the patch panel to ports on the 24-port switch.

### Step 4: Router Connection

The switch will connect to the router. The router's LAN interface will serve as the default gateway for the user computers.

### Step 5: Firewall and External Connectivity

A firewall will provide security for traffic leaving or entering the organization's network. The router/firewall arrangement will provide connectivity to the WAN/Internet.

### Step 6: IP Configuration

Each computer will receive a unique IPv4 address within the 192.168.10.0/27 network.

### Step 7: Testing

Connectivity will be verified using commands such as:

ping 192.168.10.1

and by testing communication between different PCs.

## 8. Initial Network Diagram

The preliminary physical/logical design can be represented as follows:

Internet / WAN
↓
Firewall
↓
Router
192.168.10.1
↓
24-Port Switch
↓
PC-01 | PC-02 | PC-03 | ... | PC-15

All network equipment is housed in a Network Rack.

Network Rack contains:

- Patch Panel

- Switch

- Router

- Firewall

## 9. Proposed Packet Tracer Implementation

For the practical implementation in Cisco Packet Tracer, the following devices can be used:

| Device | Quantity | Purpose |
| --- | --- | --- |
| PCs | 15 | User workstations |
| 24-port Switch | 1 | Central LAN connectivity |
| Router | 1 | Default gateway and WAN connectivity |
| Firewall | 1 | Network security |
| Patch Panel | 1 | Cable termination/organization |
| Network Rack | 1 | Physical equipment organization |

The 15 PCs will be connected to switch ports Fa0/1 through Fa0/15. The switch will be connected to the router through an appropriate Ethernet/GigabitEthernet port.

The PCs will be assigned addresses from the 192.168.10.0/27 subnet, with 192.168.10.1 configured as the default gateway.

## 10. Conclusion

The proposed network design satisfies the requirements of an organization with 15 users by providing a reliable, manageable, and scalable LAN.

The Star topology is selected because it provides good performance, easy troubleshooting, reliability, and simple expansion. A 24-port switch provides sufficient connectivity for the current 15 users while leaving additional ports for future devices.

The proposed router provides LAN-to-WAN connectivity, while the firewall improves network security. The patch panel and network rack provide structured cable management and centralized equipment organization.

The use of a single-domain environment and structured IPv4 addressing simplifies network administration and resource access. Overall, the proposed design provides a strong foundation for implementing and testing the network in Cisco Packet Tracer.